1. GFSK: From Calculus to Hardware

The goal of GFSK is to prevent the "splatter" of frequencies that occurs when you flip a bit instantly.

1.1 The Convolution of Logic and Gaussian Curves

In your report, you should explain that the Pulse Shaping Pulse $g(t)$ is the "bridge" between the digital domain (square pulses) and the analog domain.

Detailed Calculation: Why $BT=0.5$?

In BLE, the bit rate is 1 Mbps, so $T_b = 1 \mu s$.

If $BT = 0.5$, then the 3dB bandwidth $B = 0.5 / 1 \mu s = 500 \text{ kHz}$.

- Significance: If $B$ were wider (e.g., $BT=1.0$), the signal would be sharper but would interfere with adjacent BLE channels (spectral leakage).

- Significance: If $B$ were narrower (e.g., $BT=0.3$), the "smearing" would be so heavy that the current bit would completely bury the next bit (Intersymbol Interference).

1.2 Phase Accumulation (The Integrator)

The formula in 1.5 is the mathematical soul of a VCO (Voltage Controlled Oscillator) or a NCO (Numerically Controlled Oscillator).

- Calculation: For a logic '1', the frequency shift $\Delta f$ is defined by the modulation index $h$.

$$\Delta f = \frac{h}{2T_b} = \frac{0.5}{2 \times 1\mu s} = 250 \text{ kHz}$$

- Result: A '1' transmits at $f_c + 250 \text{ kHz}$ and a '0' at $f_c - 250 \text{ kHz}$. The integral ensures that when moving from '1' to '0', the phase doesn't "jump," which would cause massive RF noise.

2. CRC24: Polynomial Division in GF(2)

In your RTL, you aren't doing "long division" with numbers; you are using XOR gates.

2.1 Manual Calculation Example (Simplified 4-bit)

To explain this in your report, use a smaller example to show the mechanics before showing the 24-bit version.

- Message ($M$): 1101 ($x^3 + x^2 + 1$)

- Generator ($P$): 1011 ($x^3 + x + 1$)

- Step: Multiply $M$ by $x^3$ (shift left 3): 1101000.

- Division (XOR):

- 1101000

- 1011    (XOR)

- ----

- 0110000 -> 110000

- 1011   (XOR)

- ----

- 011100 -> 11100

- 1011 (XOR)

- ----

- 01010 -> 1010 (Remainder = CRC)

2.2 The Factor Theorem Proof

You mentioned $P(1) = 0 \pmod 2$. This is a crucial "quality check."

- Explanation: If a polynomial has an even number of terms (like the BLE polynomial which has 8 terms), then $x+1$ is a factor. In binary math, $x+1$ is the "parity bit" generator. By including it in the 24-bit polynomial, BLE ensures it never misses a single-bit error or any odd-numbered burst error.

3. Data Whitening: The LFSR Matrix

Whitening is used because a long string of zeros (000000...) in RF looks like a flat DC line, which makes the receiver's clock "lose its rhythm."

3.1 The "Why" of the Matrix

In your report, explain that the State Transition Matrix ($T$) represents the hardwired connections in your Verilog code.

- If your LFSR is $x^7 + x^4 + 1$, then bit 7 is the XOR of the previous bit 4 and bit 7.

- Hardware Mapping: Each "multiplication" by $T$ in math corresponds to one clock cycle in your BLE Baseband RTL.

4. Probability: The "Noise Floor"

The BER formula $P_b \approx \frac{1}{2} \exp(-E_b/2N_0)$ is your "Gold Standard."

4.1 Applying to your Project

When you simulate your Verilog model in a testbench:

- You inject random bit flips (simulating noise).

- You count how many packets the CRC fails to catch or how many bits are wrong.

- The Analysis: If your simulated BER is much higher than the formula, your synchronization or demodulation logic is likely inefficient.